SolarSense EMS

Full-Stack Residential EMS Audit & Next-Phase Sprint Plan (Station LHR-RES-01 / 904-A JT)

| Target System: Station LHR-RES-01 / 904-A JT (100% Residential Hybrid EMS, Lahore) | Hardware Envelope: 7.02 kWp Bifacial PV (12×585W) ¦ GoodWe ES LV Inverter ¦ 5.12 kWh LiFePO4 BESS |
| --- | --- |
| Tariff Framework: LESCO A-1 ToU & Net-Metering (46.85 PKR Peak, 34.53 Off-Peak, 27.00 Export) | Team Allocation: Abubakar (Lead — Systems/ML/MILP/API/LLM) ¦ Zaeem (CV/Next.js UI) |

1. Executive Standing & Residential Product Architecture

SolarSense EMS has officially transitioned into a 100% Residential Hybrid Solar & Battery Energy Management System anchored to real-world empirical telemetry from Station LHR-RES-01 (904-A Johar Town, Lahore, Pakistan). All historical LESCO B3 industrial microgrid simulation scripts, notebooks, and redundant configuration aliases have been cleanly quarantined into archive/legacy_industrial/.

| Architectural Layer | Subsystems & Modules | Current Status & Measured Latency |
| --- | --- | --- |
| 1. Telemetry & Data Store | Polars + PyArrow Feature Store (src/data_engineering/) | 100% Complete (0.035 ms slice) |
| 2. Soiling & Wash Engine | Perez POA + CUSUM + ROI Engine (src/soiling/) | 100% Complete (7.02 kWp / 1,500 PKR) |
| 3. Computer Vision Suite | EfficientNet-B2 + CCTV Monitor (src/vision/) | 100% Complete (95.40% Test Acc / Frozen) |
| 4. Causal ML Forecasting | Solar, Load (P50/P90) & Outage (src/forecast/, src/grid_outage/) | 100% Complete (2.72 ms 3-model inference) |
| 5. HiGHS MILP Optimizer | CVXPY DPP 5.12 kWh BESS Solver (src/optimizer/) | 100% Complete (10.45 ms warm solve) |
| 6. FastAPI Compute Gateway | REST + SSE Streaming Gateway (src/api/) | 85% Complete (Core Ready; 3 BFF routes left) |
| 7. Multi-Agent LLM Layer | Explainable Dispatch & Wash Brief (src/llm_agents/) | 0% Built (Scheduled for Next Sprint) |
| 8. Next.js 14 Web App | App Router + TypeScript + Tailwind + Zustand (frontend/) | 0% Built (Scheduled for Next Sprint) |

2. Module-Wise Completion Audit (Targeting a Full-Stack Web App)

Module 1: Data Engineering, SEMS+ Telemetry & Arrow Feature Store (src/data_engineering/) — 100% Complete

Built & Verified: EMSDataRepository, TariffPolicy.lesco_a1_residential(), OptimizationHorizonSlice, SimulationWindowDTO, and MLTaskSplits. 11,136 hourly records (464 days) anchored to GoodWe SEMS+ portal daily totals (7.02 kWp ceiling, 0.000000 kW nocturnal solar across all 5,193 night hours, 113 blackout events <= 5.12 kWh). Sub-millisecond 24-hour slice retrieval: 0.0346 ms (budget < 1.0 ms).

Remaining for Final Web Product: None in core data engineering; ready for direct consumption by FastAPI and Next.js.

Module 2: Physics Soiling & Econometric Wash Advisor (src/soiling/) — 100% Complete

Built & Verified: calibrate_soiling_model.py calibrated for 7.02 kWp (12 × 585 W), LESCO A-1 ToU rates (34.53 PKR/kWh Off-Peak, 46.85 PKR/kWh Peak), and 1,500.0 PKR residential wash cost (fitted post-wash decay k = 1.211%/day, 1,507.4 kWh cumulative loss = PKR 52,049). SolarPlantLossCalculator defaults to 7.02 kWp, 585 W bifacial modules, 1,500 PKR wash cost, and A1_ToU tariff, fixing the low-power residential derate threshold (expected_clean_kw > 0.1).

Remaining for Final Web Product: Expose SolarPlantLossCalculator via a dedicated FastAPI endpoint (GET /api/soiling/status) for the Next.js Soiling & Maintenance ROI card.

Module 3: Computer Vision Defect & Dust Inspector (src/vision/) — 100% Complete (Frozen)

Built & Verified: train_classifier.py, evaluate_classifier.py, cctv_panel_monitor.py, and models/efficientnet_b2_defects.pth. Holdout Test Accuracy: 95.40% (Macro-F1 = 0.9547 standard; 97.70% with 4-pass TTA feature fusion), zero cross-split leakage across 580 curated hybrid images.

Remaining for Final Web Product: Keep src/vision/ untouched; add a read-only FastAPI wrapper route (POST /api/vision/inspect & GET /api/vision/samples) calling CCTVPanelMonitor.predict_pil_image() for the Next.js UI.

Module 4: Causal ML Forecasting & Outage Risk Suite (src/forecast/, src/grid_outage/) — 100% Complete

Built & Verified: SolarPVForecaster (7.02 kWp clipping, R² = 0.9578, MAE = 0.1552 kW, RMSE = 0.3118 kW, Peak MAE = 0.0491 kW, 0.4183 ms latency); ResidentialLoadForecaster dual point + P90 quantile LightGBM bundle (R² = 0.8845, MAE = 0.2777 kW, RMSE = 0.3707 kW, P90 coverage = 84.7%, 1.6185 ms latency); OutageRiskPredictor Platt-calibrated LightGBM (Brier = 0.0312, PR-AUC = 0.0495, threshold = 0.0500, 0.7376 ms latency).

Remaining for Final Web Product: None in core ML training/inference; already wired into EMSDataRepository.get_optimization_slice(use_forecasts=True) (2.722 ms combined).

Module 5: HiGHS C++ MILP Battery Dispatch Optimizer (src/optimizer/) — 100% Complete

Built & Verified: CVXPY Disciplined Parametrized Programming (DPP) + HiGHS C++ solver supporting BESSConfig.station_904a_residential() (5.12 kWh capacity, 15% min SoC = 0.768 kWh, 5.0 kW max charge/discharge, 55% nocturnal blackout reserve = 2.816 kWh, and Post-Blackout Peak Recharge Suppression p_grid <= load during 18:00–22:00). Warm 24h residential solve time: 10.446 ms (budget < 25.0 ms).

Remaining for Final Web Product: None in core solver math.

Module 6: FastAPI Compute Gateway (src/api/) — 85% Complete

Built & Verified: lifespan initializes TariffPolicy.lesco_a1_residential() and BESSConfig.station_904a_residential() with 10.0 kW sanctioned ceiling in /api/backtest/stream and net-metering export vectors (p_export_kw, export_revenue_pkr) in OptimizationResponse. Measured HTTP round-trip: GET /api/window in 1.286 ms, POST /api/optimize in 13.260 ms (budget < 50.0 ms).

Remaining for Final Web Product (15%): Add 3 lightweight BFF endpoints: (1) GET /api/soiling/status, (2) POST /api/vision/inspect, and (3) POST /api/agent/advisory.

Module 7: Multi-Agent LLM Explanation & Advisory Layer (src/llm_agents/) — 0% Complete (Next Sprint)

Built & Verified: Directory src/llm_agents/ exists as a placeholder; agent responsibilities specified in docs/agents.md.

Remaining for Final Web Product (100%): Implement deterministic + LLM-backed multi-agent orchestrator (src/llm_agents/ems_copilot.py) with 3 specialized roles: (1) Grid & Tariff Arbitrage Analyst, (2) Blackout Resilience & SoC Guard Advisor, and (3) PV Health & Wash ROI Advisor (with deterministic fallback when offline).

Module 8: Next.js 14 Full-Stack Web Dashboard (frontend/) — 0% Complete (Next Sprint)

Built & Verified: Backend CORS, SimulationWindowResponse, OptimizationResponse, and SSE /api/backtest/stream payloads are shaped for 60 FPS client playback.

Remaining for Final Web Product (100%): Scaffold and build frontend/ using Next.js 14 (App Router), TypeScript (strict), Tailwind CSS, Zustand, TanStack Query, and Recharts: (1) Live Station 904-A Power Flow Deck, (2) HiGHS MILP vs. Stock GoodWe Firmware Comparator, (3) Multi-Day SSE Rolling Backtest Player, (4) Optical Defect & Soiling Wash ROI Console, and (5) AI Copilot Drawer.

3. Unnecessary / Quarantined Files Log

During Step 1 of this audit, the following redundant, duplicate, or legacy industrial files were cleaned or quarantined into archive/legacy_industrial/:

| Original Path | Action Taken | Engineering Rationale |
| --- | --- | --- |
| src/lesco tarrif.json (with space) | Moved to archive/legacy_industrial/lesco_tarrif_legacy_space.json | Unreferenced legacy NEPRA slab draft; src/lesco-tarrif.json (with hyphen) is the canonical Station LHR-RES-01 residential config. |
| src/optimization/ (__init__.py, milp_solver.py) | Moved to archive/legacy_industrial/src_optimization_alias/ | Redundant 8-line package alias duplicating src/optimizer/milp_solver.py; zero active imports across src/ or tests/. |
| scratch/ | Removed empty directory | Temporary scratch directory in repository root with zero files. |
| notebooks/panel_defect_classification_colab.ipynb & 01_industrial_load_eda.ipynb | Removed / Archived in commit 593b91d | Replaced by local CLI scripts (src/vision/train_classifier.py) and archived industrial EDA. |

4. Next-Phase Implementation Plan & Task Breakdown (Abubakar vs. Zaeem)

| Phase & Timeline | Task ID & Owner | Module / Target Files | Concrete Engineering Deliverables |
| --- | --- | --- | --- |
| Phase 1 (Days 1–2) Backend BFF & LLM Layer | Task A1 — Abubakar (Lead) | src/api/main.py src/api/schemas.py | Add GET /api/soiling/status (SolarPlantLossCalculator), POST /api/vision/inspect & GET /api/vision/samples (CCTVPanelMonitor read-only), and GET /api/dates. |
| Phase 1 (Days 1–2) Backend BFF & LLM Layer | Task A2 — Abubakar (Lead) | src/llm_agents/ems_copilot.py tests/test_llm_agents.py | Build Multi-Agent LLM Advisory Layer (Tariff Arbitrage, Blackout Resilience, PV Wash ROI) with structured Pydantic output + deterministic offline fallback; wire POST /api/agent/advisory. |
| Phase 2 (Days 3–5) Next.js 14 Web App | Task Z1 — Zaeem | frontend/ frontend/src/lib/api.ts | Scaffold Next.js 14 App Router + strict TypeScript + Tailwind CSS glassmorphism shell, lucide-react, recharts, zustand, and typed REST/SSE API client. |
| Phase 2 (Days 3–5) Next.js 14 Web App | Task A3 — Abubakar (Lead) | frontend/src/store/useEMSStore.ts | Build Zustand simulation store & 60 FPS timeline scrubber managing date selection (2025-07-01 to 2026-10-07), forecast toggles, tariff overrides, and SSE backtest consumer. |
| Phase 2 (Days 3–5) Next.js 14 Web App | Task Z2 — Zaeem | frontend/src/components/charts/ | Render 24h stacked power dispatch chart (PV, Load, p_grid, p_export, p_ch, p_dis), 5.12 kWh BESS SoC trajectory (15% floor, 55% reserve), and Peak ToU (18:00–22:00) bands. |
| Phase 2 (Days 3–5) Next.js 14 Web App | Task Z3 — Zaeem | frontend/src/components/vision/ | Build Computer Vision Defect & Soiling Wash ROI Console displaying 5-class probability bars, OpenCV HUD preview, and 1,500 PKR wash break-even calculator. |
| Phase 3 (Days 6–7) Integration & Defense | Task A4 & Z4 — Joint | tests/ & frontend/ | End-to-end latency verification (< 50 ms API round-trip), zero TypeScript/ESLint build errors (npm run build), and 100% passing pytest suite. |

5. Quantitative Acceptance Criteria & Verification Scorecard

| Subsystem / Benchmark | Target Acceptance Metric | Measured Verification Proof |
| --- | --- | --- |
| Non-CV Automated Test Suite | 100% passing (>= 87 tests, 0 failures) | 87 / 87 passed in 9.01s (.venv/bin/pytest --ignore=tests/test_train_classifier.py) |
| 24h Telemetry Slice Retrieval | < 1.00 ms retrieval latency | Measured: 0.0346 ms (28x under budget) |
| 24h 3-Model ML Inference | < 10.00 ms combined latency | Measured: 2.7220 ms (Solar 0.42 ms + Load 1.62 ms + Outage 0.74 ms) |
| 24h Residential MILP Solve | < 25.00 ms warm HiGHS solve | Measured: 10.4462 ms (5.12 kWh LiFePO4 BESS) |
| FastAPI GET /api/window | < 50.00 ms HTTP round-trip | Measured: 1.2856 ms (38x under budget) |
| FastAPI POST /api/optimize | < 50.00 ms HTTP round-trip | Measured: 13.2596 ms (3.7x under budget) |
| Next.js Production Build | 0 TypeScript or ESLint errors | cd frontend && npm run build |

6. Execution Commands Reference

# 1. Run Non-CV Test Suite (Mandatory Guardrail Command) .venv/bin/pytest --ignore=tests/test_train_classifier.py -v # 2. Calibrate Residential Soiling Model & Test Wash ROI Calculator .venv/bin/python src/soiling/calibrate_soiling_model.py .venv/bin/python src/soiling/loss_calculator.py # 3. Train & Benchmark Causal ML Forecasters (7.02 kWp Solar, Residential Load, Outage Risk) .venv/bin/python src/forecast/train_solar_pv.py .venv/bin/python src/forecast/train_residential_load.py .venv/bin/python src/grid_outage/predict_outage.py # 4. Launch FastAPI Residential Compute Gateway .venv/bin/uvicorn src.api.main:app --reload --port 8000